Device USB\VID_1D50&PID_6002\23e000064214d79547522d62c02000f5 was configured.

Driver Name: winusb.inf

Class Guid: {88bae032-5a81-49f0-bc3d-a4ff138216d6}

Driver Date: 06/21/2006

Driver Version: 10.0.19041.1

Driver Provider: Microsoft

Driver Section: WINUSB.NT

Driver Rank: 0xFF2000

Matching Device Id: USB\MS_COMP_WINUSB

Outranked Drivers:

Device Updated: false

Parent Device: USB\ROOT_HUB30\4&c7796dc&0&0

wget https://github.com/greatscottgadgets/ubertooth/releases/download/2018-12-R1/ubertooth-2018-12-R1.tar.xz

tar xf ubertooth-2018-12-R1.tar.xz

cd ubertooth-2018-12-R1/host

mkdir build

cd build

cmake ..

make

sudo make install

sudo ldconfig

Filters persist until they are explicitly cleared or the system restarts (either via ubertooth-util -r or unplug/replug). To clear a filter, use the special filter none. Example:

ubertooth-btle -t none

When filtering, previous versions of the firmware would still log all advertising packets but only follow connections based on the filter parameters. As of 2018-06-R1, advertising packets that do not match the filter are dropped.

In all sniffing modes, Ubertooth can log data to PCAP or PcapNG with a variety of pseudoheaders. The recommended logging format is PcapNG (-r) or PCAP with LE Pseudoheader (-q). For compatibility with crackle (see [Using with Crackle][]), use PCAP with PPI (-c).

Interfering with connections is a feature for causing intentional interference with newly established or long-lived connections. When this attack succeeds, the BLE connection between the central and peripheral will be terminated. Pair the -i or -I flag with -f to interfere with new connections or -p to interfere with long-lived connections. Note that causing intentional interference may be illegal in your jurisdiction. Check your local laws before using this feature.

# File:Wireshark Thermo ConnectReq.png (output from 2023 RAV4 reported 17ft away)